Session 8, Paper 19

Railway Track Ballast Condition Modelling

John Andrews1

1University of Nottingham, UK

John.andrews@nottingham.ac.uk

# Abstract

The quality of the ride on a railway line is dependent upon a number of different factors among which is the condition of the ballast.  The ballast is maintained in order to keep the ride quality within acceptable limits.  Should this quality deteriorate then the ride will become uncomfortable for passengers and, in extreme cases, could pose a safety issue due to potential derailment.  Ballast can be maintained by manual intervention, tamping and stoneblowing.  In order to make sound economic decisions regarding when and how to maintain the ballast an understanding of the deterioration process must be gained.  Ballast condition can be assessed implicitly by measuring the geometry of the railway lines.  An analysis of this data will aid in understanding the deterioration process, and importantly, the variability of the process.  Models of the deterioration process can be extended to include the effects of maintenance on the track condition.  Once this has been achieved different maintenance strategies can be investigated and conclusions drawn regarding their cost effectiveness.

The methodology presented in this paper extends previous work using statistical distributions of times to different events (geometry deterioration levels) by considering different types of maintenance strategies.  Due to their flexibility, Petri nets are used to model the combined deterioration and maintenance processes.  This avoids some of the restrictions of alternative analytical methods such as Markov techniques.

Keywords – railway track, asset management, Petri nets, life cycle analysis

# Introduction

The quality of the ride on a railway is dependent upon the geometrical properties of the rails.  This in turn is dependent upon the ballast. Should this quality deteriorate then the ride will become uncomfortable and, in extreme cases, could pose a safety issue due to potential derailment.  Poor ride quality is rectified by processes such as tamping and stoneblowing which adjust the ballast under the sleepers on the track.  In order to make sound economic decisions regarding when and how to maintain the ballast, a model can be used to investigate effects of the degradation and intervention activities on the track geometry measurements.

The are several models which have been developed to predict the condition of the track geometry for use as a support tool for track asset management.  These models can be classified as deterministic or stochastic.  The deterministic models [1-7] develop an approach by which, usually through experimentation, the performance of the track is related to the factors which affect its deterioration.  A disadvantage of this type of approach is that given the same track, used and maintained in the same way, and operated with the same loads, the rate of degradation will vary.  Understanding this variation is important in the decision making process for effective asset management.  Deterministic models also suffer from the potential to have missed an important factor in the causality of degradation which invalidates the models.  Stochastic models [8-12] are usually statistical treatments based on historical records and data.  They can be used to understand the distribution of times to the degradation events and can therefore predict performance.   Such models are based on what has actually happened and account for the variability through the use of probability.  This paper proposes such a stochastic model based on the Petri net method which enhances the flexibility of the approach compared to the earlier alternatives developed.

# Petri Net Methodology

The Petri net (PN) method provides a flexible modelling approach to many practical problems experienced in business and industry.  Its flexibility has enabled it to be successfully applied to many situations encountered in system reliability problems [13].  In this paper the flexibility of the method is exploited and, with a number of enhancements to the original modelling methodology, it is applied to the asset management of railway track.

A Petri net is a directed graph has two types of node, places and transitions, which are linked by directed edges.  The places are connected as inputs to a transition, and the transitions are linked via edges to their output places, as shown in Figure 1.  The places each represent a particular state of the system whose occurrence is indicated by the presence of one or more tokens located in the place.  Transitions move the tokens from input places to output places and in this way represent the dynamics of the system as it changes state.

Figure 1a					 			Figure 1b

Figure 1:  Petri net features

There are rules which move the tokens round the Petri net by firing the transitions.  Each edge on the Petri net has a multiplicity indicated by a number associated with the edge.  Where no number is stated the multiplicity is 1.  So in Figure 1a, all edges have a multiplicity of 1 except the second input place edge which has multiplicity of 2.  The transitions are said to be enabled when there are the multiplicity of tokens in each of the input places.  Once enabled the transition will fire after the delay time t.  On firing the multiplicity of tokens are taken from each of the input places and the multiplicity of tokens are added to the transition output places.  The transition time is sampled from the appropriate distribution.  A second edge type, the inhibit edge, is also used in the Petri net models.  An example of an inhibit edge is shown in Figure 1b and has a round end rather than an arrow.  When the multiplicity of tokens exist in the inhibit place then the transition it is connected to is inhibited and cannot fire until the tokens are removed from the linked input place.

In the track model presented there are a number of non-standard transitions that have been used to make the model development more efficient.  These are described below:

Place conditional transitions (PCT)

These are transitions where the distribution from which the transition times are sampled is dependent upon the number of tokens in a specified place on the PN.  The place on which the transition depends is linked to the transition by a dotted arrow in the diagrams.  This is used to determine which distribution the transition delay related to the geometry degradation is sampled from as it is dependent upon the number of interventions carried out on the section previously.

Transition conditional transition (TCT)

These are similar in concept to the PCTs with the time distribution dependent upon the transition time of another transition in the PN (again the link between the relevant transitions is by a dotted arrow).  This can be used in the renewal part of the network where the ballast renewal is instigated as soon as the time between interventions becomes less than a set threshold.

Reset transition (RT)

When the reset transition fires it resets the tokens in part of the network to their initial places.  This can be done using the conventional PN features but would require many transitions to be added to the PN for this housekeeping task rather than representing the dynamics of the system.  This adds considerably to the complexity of the diagram making it more confusing.  Associated with the RT are a list of the places which will be reset and the number of tokens they will contain after the reset.  Resets are issued in the track model when a ballast renewal is carried out or an intervention is completed.

# Track Geometry Quality Measurements

The quality of the railway track is established by taking measurements of its geometry.  A measurement train passes along the track at specified time intervals and (on the UK railway network) records for each 1/8th mile section:

- Vertical geometry (top) - rail height standard deviation (SD) (left, right and mean), subtracted from a 35 m (short-wave) or 70 m (long-wave) running average.

- Lateral geometry (alignment) - rail alignment, averaged over the left and right rail, expressed as short and long-wave SDs.

- Gauge - rail spacing compared to the standard gauge.

- Cyclic top - a measure of resonant dip frequencies (which may cause a train to “bump” along the track, possibly leading to a derailment).

- Twist - a measure of cant variation over 3 m and 5 m.

These measurements have to be within certain tolerances to give a safe and smooth ride.  Should they fall outside the acceptable limits then maintenance is implemented.  In severe cases speed restrictions or line closure are imposed until rectification work is carried out.

The condition of the track geometry is improved by either manual interventions or, more commonly, by the use of specially designed machines called tampers or stoneblowers.  The two machines use different approaches to rectify the geometry.  A tamper moves along the track lifting the rails and inserting vibrating tines either side of the sleeper.  It then pushes the ballast under the sleeper.  Whilst rectifying the immediate problem a disadvantage of tamping is that it damages the ballast when the tines are inserted and the track geometry deteriorates faster following its application.  A stoneblower takes a different approach.  It lifts the sleeper and inserts fines under it to achieve the required adjustments.

# Track Specification

The railway track to be modelled in this research is an 1/8th mile section.  By monitoring the performance over time of such sections of the railway, degradation time distributions have been obtained.  These distributions will give the time to any level of degradation specified, as defined by the standard deviation (SD) of the track geometry measurements.  The distributions are also dependent upon the number of interventions (I) performed on the track since the ballast was last renewed and follow a Weibull distribution W[β(SD,I),η(SD,I)] with shape factor β and characteristic life η.  The critical measurement considered to indicate the quality in this research is the top measurement.

FTamp(t) and FSB(t) are the distribution of times to rectify a problem once it has been indentified using the tamper and the stoneblower respectively.  They are a function of the priority of the work which is either normal or high if the current state of the track requires the imposition of speed restrictions or line closures.

The measurement train passes along this section of track every month (30 days).  At this point in time any sections of track whose condition needs attention are identified.  There are thresholds specified for the SDs which will trigger maintenance before the restrictions are placed on the track usage σthresh.  A speed restriction will be imposed if the SD exceeds σSpeed  and a line closure at σClose.  In this model we have also applied another threshold for the SD values, σOpp this is a value less than σthresh for which maintenance is not normally carried out on a section but will be required in the near future.  When this limit is exceeded maintenance will not be requested but will be carried out if a track section in the neighbourhood is being maintained and the machines are available.  This is opportunistic maintenance.

The maintenance strategy is that there are two tamping machines and one stoneblower on this part of the network.  The preferred means of maintenance is tamping and when needed this will be carried out by the soonest available tamper unless this cannot be completed within 100 hours.  If this is exceeded the first available of all machines is used.  This is also true when priority maintenance is required.  Once a line has been stoneblown it cannot then be tamped until renewal of the ballast has taken place.

Ballast renewal is performed every 5 years.

# Petri Net Track Model

The Petri net model to predict the performance and life cycle costs of the track section will be described as a series of sub-models each reflecting part of the degradation, inspection and intervention activity.

Geometry Degradation

The first part of the model is shown in Figure 2

Figure 2: track degradation model

The ballast condition is in the renewed state P1 at the start of the modeling period and so a token is reset in this place.  From this state the ballast can be seen to degrade to 4 further states each reflecting a worsening condition of the track geometry.  When the condition has reached σOpp, opportunistic maintenance on the section will be permitted when a tamper or stoneblower is available in the area.  This is represented by place P2.  Further degradation will bring the track to a state where the SD exceeds σthresh (P3) at which point maintenance is scheduled for this section.  It is anticipated that the maintenance will be carried out prior to achieving the conditions σSpeed  or σClose (P4 and P5).  The transitions between these states are stochastic with the times being sampled from the appropriate distributions.  Distributions can be derived from historical data to give the time to deteriorate to a given SD value from new.  Therefore the transition time from P1 to P2 (T1) can be sampled directly from these distributions, but additional processing is required to give the times to degrade from one specified state to another in terms of the distributions to reach these states from new as is required for T2-T4.  The method to do this is given in [14].

Geometry Inspection Process

The inspection process which represents the measurement train passage along the appropriate track section is illustrated in Figure 3.

Figure 3: track inspection process

P10 and P11 represent the inspection cycle of the measurement train. P10 contains a token when the train is not on the current section and the token transfers to P11 at regular intervals of  θ (T5) when inspection is taking place.

When a deteriorated state of the track exists (tokens in P3, P4 or P5) coinciding with a time of inspection (token in P11) the appropriate transition (T8 for P3) is enabled and fires immediately placing tokens in places P7, P8 and P9 for states P3, P4 and P5 respectively.  The transition from P3 to P7 is illustrated in figure 3 the others follow the same principles. Once the token is placed in P7 to indicate that the degraded state is now known, tokens are returned to the actual state and the inspection process nodes, P3 to enable further degradation of the geometry prior to the intervention, and P11 to continue the inspection cycle.  A token is also placed in node P13 to prevent repeated firing of the transition and shortly after the token is returned to P11, an infinitesimally short time ε, it is moved on to place P10.

Figure 4: maintenance request

Maintenance request

Once a token is in any of the places P7, P8 or P9 then maintenance is scheduled.  The priority of such a requirement is dependent upon which of the degraded states is known.  For P8 or P9 when a speed restriction or a line closure is imposed then the maintenance is top priority.  For P7 a normal priority request is issued.  This section of the Petri net is illustrated in Figure 4.  Tokens from P7, P8 and P9 cause transitions T9, T10 and T11 to immediately fire.  These place a token in P16 to request maintenance and P17 if the maintenance request is urgent.

Selection of Maintenance Machine

As soon as a token is placed in P16 the decision has to be made as to how the maintenance will be carried out.  For normal priority maintenance then the rule will be that tamping will be preferred and that the tamping machine which can do the job the quickest will be selected unless the time awaiting maintenance surpasses 100 hours and then the first available tamper or stoneblower is selected.  For priority maintenance the fastest means of repair is always selected.  Figure 5 shows that this is achieved on the PN by a token in place P17 indicating a priority situation which suppresses transition T15 and enables transition T14.  In the event that tampers 1 or 2 can do the intervention fastest and prior to 100 hours then transitions T12 or T13 will fire and select the appropriate tamper.  For priority maintenance transition T14 is also enabled to allow the selection of the stoneblower if this provides the fastest means of repair.  For normal priority if the time for maintenance exceeds 100 hours then T15 fires and the first available machine is selected through transitions T17-T19 to carry out the work.

It should be noted that transitions T12, T13 and T14 are place conditional transitions (PCTs)  where the transition time is sampled from a different distribution depending upon the presence of a token in P17 or not.  The times sampled are the times to get the machine to the location and to carry out the restoration of the track geometry.

Figure 5: maintenance machine selection

Once stoneblowing has been used on any section then tamping should not be carried out.  Place P23 contains tokens to indicate the number of times stoneblowing has been performed on this section since the ballast was renewed.  A token present in this place suppresses any transition which would select a tamper for maintenance until the ballast renewal has taken place.

Maintenance of the track Section

Once tokens exist in places P19-P21 in Figure 6 it indicates that the simulation has progressed to the point where maintenance has been carried out restoring the track geometry and it indicates which machine has been used.  The track geometry will not be in the ‘as good as new’ state but it will be better than it was prior to the maintenance.  Its future deterioration will depend on the number of interventions that have now been performed on it.  As such the new track condition is represented by Place P22 on the Petri net and the number of tokens in places P24 and P23, which record the number of tamping and stoneblowing actions carried out respectively, are updated to record the additional intervention that has taken place.  From the state represented by a token in place P23 the condition will again deteriorate over time to the state represented by P2.  the transition for this process, T23 is again a place conditional transition (PCT) dependent upon the intervention history given by tokens in places P23 and P24.  All degradation transitions T2, T3 and T4 in Figure 2 will also be dependent upon P23 and P24. Note that transitions T20-T22 are also reset transitions where any tokens in the PN which currently reflect the section condition or the maintenance process are removed.

Figure 6: maintenance completed

Ballast Renewal

For this model the renewal process has been assumed to be very simple and carried out after a fixed ballast life of L.  This is illustrated in Figure 7.  Note that transition T25 is a reset transition which re-initialises all places which represent the actual and known geometry conditions and the details of the interventions that are in progress.

Figure 7: ballast renewal process

Complete track Section PN

Putting all of the sub-models together gives the overall PN structure shown in Figure 8.

Figure 8: Full section PN model

# Model Analysis and outputs

The PN is quantified by carrying out a Monte carlo simulation [15] for many track lifetime periods until convergence is achieved.  The model can be used to calculate:

- system availability

- number of train delays

- delay time distribution

- distribution of the number of times maintenance has been carried out

- the maintenance resources used (cost, man effort, testing machines, maintenance machines, spares)

- component lifetime lost through premature replacement of components

- number and time distributions for both unscheduled and scheduled possessions

- number and type of safety related failures

Opportunistic maintenance

When the track sections are linked together to model larger sections of the railway network then the capability to consider opportunistic maintenance, built into the model described, can be used.  Opportunistic maintenance would be carried out when a token was in place P2 on one section of the track and a token in place P19-P21 on the neighbouring section.  Extra transitions would then be added to link the two section PNs and enable the opportunistic repair and update the number of tokens for the appropriate type of intervention.  The parts of the sections which represent the passage of the measurement train (P10-T5-P11-T6-P10) would be common and only exist once on the combined network.

# Summary & Conclusion

This paper describes the development of a Petri net model for use in the asset management of railway track ballast. An indication of the outputs from the model analysis are provided.  The discussion highlights how the model built can be used in its own right or as a sub-model in a more complex network or line based representation of the system.

Acknowledgement

John Andrews is the Royal Academy of Engineering and Network Rail Professor of Infrastructure Asset Management. He is also Director of The Lloyd’s Register Educational Trust1 Centre for Risk and Reliability Engineering at the University of Nottingham. He gratefully acknowledges the support of these organizations.

1(The Lloyd's Register Educational Trust (The LRET) is an independent charity working to achieve advances in transportation, science, engineering and technology education, training and research worldwide for the benefit of all)

# References

- Shenton M.J.,  Ballast deformation and track deterioration, Track Technology, Tomas Telford Ltd, p253-264, 1985.

- Chrismer, S. and Selig, E.T., Computer model for ballast maintenance planning, Proc 5th Int heavy haul railway conf, Beijing, p223-227, 1993.

- Sato, Y., Optimum track structure considering deterioration in ballasted track, Proc of int heavy haul conf, Cape Town, South Africa, p576-590, 1997.

- Hamid, A. and Gross, A., Track-quality indices and track degradation models for maintenance-of-way planning, Transportation research Board, 802, p2-8,1981.

- Bing, A.J. and Gross A., Development of railway track degradation models, Transportation research Board, 939, p27-31, 1983.

- Sadehghi, J. And Askarinejad, H., ‘Development of Improved Railway Track Degradation Models’, Structure and Infrastructure Engineering, Vol 6, No 6, pp675-688,2010.

- Shafahi, Y. and Hakhamameshi, R., Application of maintenance management model based on Markov chain and probabilistic dynamic programming for the Iranian railways, Trans A: Civil Engineering, Vol 16, No 1, pp87-97, 2009.

- Lyngby N., Hokstad, P. and Vatn, J., RAMS Management of Railway Tracks, published in the Handbook of Performasbility Engineering (Ed Krishna Misra ), Springer Press, 2008, pp1123-1145.

- Podofillini, L., Zio, E., and Vatn, J., Risk-informed optimisation of railway tracks inspection and maintenance procedures, Journal of Reliability Engineering and System Safety 2006, 91, 20–35.

- Kumar, S., Espling, U., and Kumar, U., A holistic procedure for rail maintenance in Sweden, Proc. of the IMechE, Part F: J. Rail and Rapid Transit 2008, 222 (4), 331-344.

- Quiroga, L.M., and Schnieder, E., ‘Monte Carlo Simulation of Railway Track Geometry Deterioration and Restoration’, to be published in the Journal of Risk and Reliability, Part O of the Proceedings of the IMechE.

- Quiroga, L.M., and Schnieder, E., ‘A Heuristic Approach to Railway Track Maintenance Scheduling’, WIT Transactions on The Built Environment, Vol 114, pp687-699, 2010

- Schneeweiss, W.G.,  Petri Nets for Reliability Modelling, LiLoLe- Vrelag Publishing Company Limited, 1999.

- Andrews, J.D., A Modelling Approach to Railway Track Ballast Asset Management, submitted to Proceedings of the IMechE, Part F.

- Andrews, J.D. and Moss, T.R, Reliability and Risk Assessment, Second Edition, Professional Engineering Publishers, Ltd, 2003.