Session No.3, Paper No.8

A new Methodology to forecast Reliability and to determine appropriate maintenance

J. de Reffye,

PI-RAMSES Consulting

10, rue Carnot, 78000  Versailles

dereffye.pi-ramses@sfr.fr

Abstract

The usual methods used to evaluate the parameters of reliability of a system are based on the FMECA FTA approach and Petri nets simulation.  The new method proposes:

- To estimate the parameters of intrinsic reliability (without maintenance) by physical-chemical models of behaviour

- To estimate the parameters of extrinsic reliability ( with maintenance) by statistical processing of experimental data

- To process a data fusion between the two approaches of the reliability

- To fit the maintenance operations using the preceding calculations of the reliability.

The application of the strength-stress method allows the forecasting of the failure probability of each component and it can be propagated through the failure trees. The physical approach to analyze the reliability of the system can be improved using a statistical approach : the data empirically gathered are used through a method of data merging in combination with the numerical results obtained by the physical approach.  This methodology can be compared with a Bayesian approach as long as we identify the physical results as an a priori statistical law.

The physical data are typically provided by the manufacturers while the statistical data related to the product are provided by the customer's maintenance and warranty departments.

We are interested by the maintenance policy to put in its place for the respect of the requirements of the extrinsic reliability and the availability objectives.

This maintenance policy is defined by the joint preventive and corrective maintenances.

The set of maintenance operations is now considered as a mathematical operator which transforms the values of intrinsic reliability in extrinsic reliability.

The tool of analysis is sufficiency powerful to control the efficiency of the maintenance and to minimize its cost.

Keywords

I	INTRODUCTION TO THE METHODOLOGY

- The general methodology to optimize the maintenance is defined by :

- The knowledge of the prediction of the intrinsic reliability supplied by modeling and trials

- The knowledge of the extrinsic reliability supplied by the observation of the return of experiment.

The set of results is merged according to the degree of uncertainty of the obtained data or estimations.

The different steps of the methodology are described below.

- Estimations of the predicted intrinsic reliability

The predicted intrinsic reliability which is the reliability without maintenance is based on the failure probability of the components calculated mode by mode by means of a strength-stress applied on the mechanical components as the electronic components with a generalization of the stress-strength method ([1])

- The stress-strength method uses the physical – chemical parameters of the strength opposite at the physical stresses and the chemical damages.     These parameters as the strengths are uncertain. The environmental conditions are imprecise and the set of parameters and conditions of using are moving with the time.

- The intrinsic reliability is equally based on the results of the trials of strength and hardiness realized on the components this trials being defined by normalized procedures.  The failures as the modes of failures what are causes of failure are noted.   The obtained result is a failure rate with an interval of confidence.   The law of reliability of the component is calculated taking on account the interval of confidence and the result is merged with this one obtained by the stress-strength modeling approach.

- Merge of the two estimations of the intrinsic reliability

The two laws are merged with the following process :

- The trials supply generally a constant failure rate because the trials time is short.  One tests the possibilities to exceed of the limite states. The estimation of the intrinsic reliability by means of modeling supplies a time varying failure probability. The values of reliability by trials are used to adjust the reliability by modeling at the time origin.

- Once the failure probabilities is calculated for each component and for each failure mode one calculates the probability to lose a function by the use of the dysfunctional analysis applied to the functional trees.

- Building the Bayesian net.

Going from the functions to the states is possible if theses ones are previously defined.  For doing each state is defined by means of the lost external functions and the internal functions what are need to realize it. The nominal state is the good working one. The damaged states are defined by the lost of external functions. The numbers of internal functions what are necessary to realize the external function allows calculate the transition probabilities between states.

Indeed from the transition probabilities one obtains the transition rates            and from the transition rates one obtains the probabilities of realization of the different states resolving the Kolmogorov’s equations.

The bayesian net related to the system is completely defined.

- Processing of the data of the return of experiment

The observed failures are due to the lost of external functions. One observes consequently the transitions between states from the nominal state. It is possible to go from the states to the components through the failure trees using a linearization method associated with a likelihood function . This process is to determine the most likely component cause of the failure.

The result is generally given as a failure rate estimated from the time serie of the counting of failure events what are of same type ( time derivative of the number of failures versus the time) . At this rate one associates an error interval of the estimation method. This interval is supposed to represent the standard deviation of a random variable.

- At this step of the methodology the failure rate is the rate of realization of the different states of the system with maintenance and this rate is conditioned by the different operations of maintenance set to work.

Because of the estimation of the forecast reliability is calculated without maintenance the comparison with the values from the REX must take on account the influence of the maintenance.

So it is necessary to define the operator of maintenance ( at the mathematical sense) and characterize its properties. If it would be invertible one could be placed on the values of intrinsic reliability.

Unfortunately it is not because of the truncation of the probability laws of failure due to the time finite interval between maintenance operations what implies that its kernel is no degenerated. This operator is not continue at zero and no invertible consequently.

- So the merging between the failure probabilities obtained by the joint values from trials and models and the REX values must be made in the domain of extrinsic reliability which is this one of the REX values. Hence it is necessary to transform the values of intrinsic reliability to values of extrinsic reliability applying the maintenance operator on the forecast failure probabilities obtained by the models and trials.

- Merging of the REX data with the forecast extrinsic reliability

The maintenance operator is applied to the values of the forecast intrinsic reliability

The values from the REX are compared to the forecast ones.

The proposed method to merge the two groups of values is a recursive merging of the failure rates. The logarithmic derivative of the failure probability gives the failure rate.

- After the merging the failure probabilities are obtained by integration of the exponential of the rates.

At the end of this step the probabilities of realization of the states are therefore estimated from physical-chemical models trials of hardiness and data of return of experiments. All sources of information are taken on account.

- Analysis of the maintenance

The maintenance is considered as a mathematical operator. We are developing this point of view.

The maintenance is split on a preventive maintenance and a corrective conditional maintenance

- The preventive maintenance is defined by interventions made at regular time intervals.

Mathematically it is the application of a « Dirac’s comb » to the probability law of the failure of the component replaced at each preventive maintenance operation.

- The corrective conditional maintenance is defined by visits or inspections made at regular dates but the decision to replace a spare is depending on diagnostic of a maintenance agent or an automaton of diagnostic.

The tests at the start of a system belong to this category. These ones what detect a failure on line do not belong to this category but at the process outage – repair. This process defines the observed extrinsic reliability.

- Calculation of the failure probability of components with maintenance

The objectives of extrinsic reliability and availability are given as strengths to be respected. The extrinsic reliability is correctly estimated. So it is possible to define the maintenance according to the gain of reliability and the percentage of working time which will be used for the maintenance.

The preventive maintenance cannot make an error while the corrective conditional maintenance can do it according to the first and second kind error :

- Do not change a component while it could be overtaking a limit state before the next inspection ( first kind risk : no detection)

- Change a component while it could be staying under the limit state ( second kind risk : false alarm)  .

The first kind risk implies a memory effect which can be limited as soon as the no detection probability is lower than a fixed threshold.

The calculation of the failure probability of a component after preventive maintenance is easily made noting its perfect regeneration because the component is ipso facto new after replacement and the duration between two preventive maintenances.

The calculation of the failure probability of a component after corrective conditional maintenance is more complicated because the quality of the detection tests must be taken on account.

In any case the failure probability of a component is obtained by the product of the failure probability with the preventive maintenance times the failure probability of the component with the corrective maintenance.

- Respect of the requirements of reliability and availability : adjustment of the maintenance

The product of the maintenance operators ( preventive and corrective conditional) supplies the conditional failure probability of the component with its maintenance.

As we know the gain of reliability to be reached to satisfy the requirements the maintenance can be adjust.

The preventive maintenance is worked during the request periods at maximum. The corrective conditional maintenance is worked maximally equally during the request periods of the system except the tests on line for the knowledge of the state of the system before using.

The duration of scheduled unavailability due to the preventive maintenance the duration of no scheduled unavailability due to the corrective conditional maintenance and the duration of the repairs of outages are known.

The intrinsic reliability is known.

The requirements of reliability and availability are known.

It is therefore possible to put completely the associated problem of mathematical programming. Solving it is a problem of operation research.

2	CANONICAL EXAMPLE

This canonical example is extracted from the work of a steam locomotive. A steam locomotive is old fashioned! But the mechanical problems of this work solved in the early XX° century are really up to date. The presented model is the four-cylinder speed engine built for the Great Western Railway british company :

Figure 1 : The king class locomotive

In this example we are just interested by the system rod-crank : the steam produced by the boiler is introduced in the cylinder and pushes the piston. The piston is linked to the motive rod by a guiding groove and the motive rod is linked to the wheel by a crankpin. We do not take on account the other rods and the distribution system which imbedded in this engine. To apply the stress-strength method at different components it is necessary to define the strengths and the stresses. These ones are due to mechanical strengths what are the results of the second Newton’s law. So it is necessary to modeling the moves for obtaining the accelerations.

2.1		DEFINITION OF THE DIFFERENT WORKING STATES OF THE ROD-CRANK SYSTEM

This part is defined by the engineering : what kind of degradation damage and failures are admissible and when do they realize ?

In this example we decompose the work of the system in four states:

- Nominal state: perfectly working conditions

- Damage state # 1: deformation of the shaft of the piston due to an increasing of the stresses due to the extra rubbing due to a lack of lubrication and an increasing of the working temperature

- Damage state # 2: deformation of the motive rod due to an increasing of the stresses due to the extra rubbing due to a lack of lubrication and an increasing of the working temperature

- Absorbing state: deformation of the shaft of the piston and deformation of the motive rod. Break down of the system by excess of stress implicating the break of the shaft of the piston or the break of the motive rod.

The five components are:

- C1 : the shaft of the piston

- C2 : the groove

- C3 : the rod

- C4 : the crankpin

- C5 : the motive wheel

The four functions are

- F1 : to move the shaft guided by the groove

- F2 : to move the motive rod by the shaft of the piston

- F3 : to move the crankpin by the motive rod

- F4 : to move the motive wheel by the crankpin

We represent the links between components functions and states by the following scheme:

Figure 2: logical scheme of the reliability of the system

2.2		STATE PROBABILITIES AND BAYESIAN NET

- We achieve the method to calculate the probabilities of realization of states the transition probabilities between states and the completion of the Bayesian net.

2.2.1	Failure modes

The failure modes are deduced from two ones:

- Mechanical ageing with deformation due to the decreasing of the mechanical parameters (RP02) according to the Wöhler’s curve and to the increasing of the temperature with the reaching of a limit state (overcrossing of the Rm parameter).

- Bad lubrication with deformation due to the decreasing of the mechanical parameters (RP02) according to the Wöhler’s curve and to the increasing of the temperature with the reaching of a limit state (overcrossing of the Rm parameter).

2.2.2	Calculation of the state and transition probabilities of the Bayesian net

The transition probabilities of the Bayesian network defined by the 4-state graph and the rates of transition between the corresponding states are given by:

λN,1(t)=1proba(F1∪F2)ddtproba(F1∪F2)  λN,2(t)=1proba(F2∪F3)ddtproba(F2∪F3)

λ1,A(t)=λN,2(t)  ; λ2,A(t)=λN,1(t)		[1]

The probabilities of realization of the different states are given by the Kolmogorov’s equations [2]. And the Bayesian network is completely defined by:

      - The state graph

      - The transition probabilities

      - The state probabilities.

2.3		DEFINITION OF THE MAINTENANCE TO HOLD AN EXTRINSIC RELIABILITY

In this example we suppose that the maintenance is configured at two levels:

- Preventive maintenance: overhaul of mobile parts every x miles or by means of the rolling law every T.

- Corrective conditional maintenance: monitoring and lubrication of mobile component parts before each run.

To respect the requirements of reliability and availability it is compulsory to calculate the failure probabilities of failure modes.

Let be the following figure :

Figure 3 : scheme of the set of motive parts

- F is the applied strength by the rod to the groove

- Ff is the rubbing strength

The rubbing coefficient is defined by:

Cf=FfmaxFN

Where Ffmax  is the maximum strength before slipping. This one appears if the following inequality is respected:

Fcosα≥Ffmax=CfFN=CfFsinα

So :

1tgα≥Cf   ;   tgα≤1Cf  			[2]

Until the lubrication is normal one easily demonstrates that the strengths are very lower than the limit states.

In the opposite case when an anomaly of slipping suddenly appears if we admit a rubbing coefficient approximately proportional to the applied strength the previous inequality becomes:

(F+ΔF)cosα≥Ffmax=(Cf+ΔCf)(FN+ΔFN)=(Cf+ΔCf)(F+ΔF)sinα

tgα≤1(Cf+ΔCf)

For a whole work the following statistical laws must be defined :

- Values of lubrication of the used oils

- Values of the rubbing coefficient

- Quality of the monitoring and light maintenances before starting (tests of the state of mobile parts and oiling)

But for simplifying the problem we take on account only the applied strength and the rubbing coefficient. We suppose that they are exponentially distributed.

Two possible failures with the same failure mode ( bad lubrication) :

- Blocking of the groove and breaking of the groove – rod binding and breaking of the rod

- Breaking of the rod before the blocking of the groove

- First failure type : the inequality :

tgα≤1(Cf+ΔCf) 			[3a]

is not respected. The failure probability is the probability that this inequality is not.

- Second failure type : The strength on the rod compression or tensile is equal to F/Sb

(where Sb is the section of the rod) . The failure probability is the probability that the stress is over the limit of the compression or the tensile.

Pf=P(FSb>RP02)				[3b]

where RP02 is the deformation limit.

Because of the damage phenomena the failure probability is an increasing time function, from 0 to 1.  Presently the groove and the rod are supporting cyclic loadings one cycle one wheel turn.

- Let Pf10(t) respectively Pf20(t)  the probability of the first failure type respectively the second failure type in normal working conditions (correct lubrication)

- Let Pf1(t) respectively Pf2(t)  the probability of the first failure type respectively the second failure type in abnormal working conditions (no correct lubrication)

- Let P0 The no-detection probability of a default on a mobile part or on the oiling process.

One supposes that N visits are between two preventive maintenances and that the two maintenances are spaced of T as previously put.

As the P0 and NP0 are generally little before 1, the probability of continuous error between two preventive maintenances is 1 – (1-P0)N  NP0 and the failure probability is bounded by :

.(1−NP0)(Pf10(ΔT)+Pf20(ΔT))<Pf(ΔT)<NP0(Pf1(ΔT)+Pf2(ΔT))  [4}

If Pf1(T) or Pf2(T) is too strong finer intervals exist. Suppose that the requirements on the reliability is a rate of failure equal to r and a rate of availability equal to ar  with the time part on the operational using of the engine equal to t/t  where t is the mean duration of the operation of preventive maintenance

We have to verify the system of inequalities:

P1(ΔT)+P2(ΔT).NP0ΔT<λr       ;   	ΔtΔTδtΔt=δtΔT<ar					[5}

2.4		MERGING THE PHYSICAL FORECAST RELIABILITY WITH THE DATA OF RELIABILITY GIVEN BY THE REX

We suppose that the observed failures are classified according to the lost functions.

We suppose that we have been able to estimate their rate of failure:

λf1(t)  ;  λf2(t)  ;  λf3(t)  ;  λf4(t)							[6]

We suppose that the first order approximation of the exponential function is valid. So according to the figure 2 and the formula 1 we obtain :

λN,1(t)=λf1(t)+λf2(t)λN,2(t)=λf2(t)+λf3(t)λ1,A(t)=λf2(t)+λf3(t)λ2,A(t)=λf1(t)+λf2(t)													[7]

The physical estimation and the statistical estimation are noisy:  The uncertainties in the physical parameters on the one and the errors of measurements in life or in trials on the other one. The general method to merge the two estimations is the data fusion which the general principle is as below :

Let DA and DB two data with degree of confidence equal to DA and DB respectively. We merge the data on inverse reason of their degree of confidence.

As example the easier way is to build this weighted sum:

DAB=σDBDA+σDADBσDA+σDB													[8]

This relationship can be applied to the previous formulations. The aftermaths of the calculations are the same. It exists also other technical ways of data fusions what are more fine but it is not the goal of this study to make a lesson about their. Some papers about are available in the signal processing area.

### 3   CONCLUSION

So it is possible to estimate the reliability of a system without any return of experiment. This approach is very useful for the new systems to estimate their reliability and fit the maintenance operations at the beginning of using. This approach is very analytic and it is rigorous in the chain of calculation. The parameters of strength in the strength-stress method are well known and given by the manufacturer and the stress parameters are deduced from the conditions of use and environment. As a data of REX are obtained, a data fusion between the reliability estimated by the physical approach and the statistical approach is the best approach to merge the two sets of data the first one by the physical parameters with this methodology and the second one by the return of experiment and a statistical processing. The maintenance operations are adjusted with the experimental knowledge increasing with the time.

4	BIBLIOGRAPHY

[1] : Jérôme de Reffye : Reliability of the power electronic components by their dynamical simulation in real working conditions, ESREL 2011 , UTT, Troyes.

[2] : W. Feller : An Introduction to the probability theory and its applications , John Wiley and Sons, 6th edition